Handout – Working with the Trotec Laser Cutter

1. Supported File Formats (Trotec & Ruby)

The following file formats are supported for import into Trotec Ruby:

Vector graphics (for cutting & engraving)

- .SVG – Scalable Vector Graphics

- .DXF – AutoCAD Drawing Exchange Format

- .AI – Adobe Illustrator (best compatibility up to version CS6)

- .EPS – Encapsulated PostScript

- .PDF – Recommended if vector-based

Raster graphics (engraving only)

- .JPG / .JPEG

- .PNG

- .BMP

Important: Cutting operations always require a vector file.

2. Software for Creating Vector Graphics

| Program | License | Advantages |
| --- | --- | --- |
| Inkscape | Free (Open Source) | Standard among makers, perfect SVG export |
| Adobe Illustrator | Paid (subscription) | Industry standard, best compatibility |
| Fusion 360 | Free for hobby/education | Ideal for technical drawings and precise measurements |

Recommendation for beginners:

Inkscape – free, reliable, and exports perfect SVG files for Trotec Ruby.

Important:

Do not switch between programs within the same design!
(Scaling and size information will no longer match.)

3. Why Different Lenses?

Short Focal Length – e.g., 1.5" CO₂ Lens

- very small focal point → fine lines, high detail accuracy

- suitable for thin materials (3–5 mm), paper, engravings

Medium Focal Length – 2.5" CO₂ and 2.85" flexx Lens

- universal use

- ideal for wood sheets, acrylic, standard cuts, large engravings

Long Focal Length – 4.0" CO₂ Lens

- larger focus area → straight cut edges on thick materials

- suitable for acrylic from ~10 mm, signage

Reminder:

The thicker the material or the deeper the cut, the longer the lens’s focal length.
Example: focal length = 1.5"

4. Why Different Nozzles?

Nozzles are part of the air-assist system and influence cut quality and safety.

Small Nozzle

- concentrated airflow

- prevents flame formation

- ideal for fine details, thin materials

Large Nozzle

- higher air volume

- cools material more effectively

- better for thick wood, cardboard, acrylic

Effect of Air Assist:

Airflow reduces smoke, cools the material, and improves cut quality.
Refer to the engraver’s handbook to see which nozzle is recommended for which material.

5. Workflow in Trotec Ruby – Short Overview

Ruby is Trotec’s modern laser cutter workflow software. Process:

1. Import File

- Drag & drop SVG, PDF, DXF, AI, etc.

- Ruby automatically detects cut/engrave objects.

2. Prepare Graphic

- Assign colors (color code = processing type)

  - Red = Cutting (RGB: 255,0,0 min. line thickness: 0.3 mm)

  - Black = Engraving (RGB: 0,0,0)

- Check the size of the design

3. Select Material & Parameters

- Choose material from the Trotec database

- Adjust power/speed/Hz if necessary
→ Always perform a test cut + test engraving beforehand. Set the power correction for engraving to at least 30%!

- Verify lens and nozzle selection
→ Installed lens must match the lens selected in the software

4. Position & Start

- Place the object on the virtual laser bed (white working area)

- Set focus

- Start the job in the software